- UBIFLEX

- INSTALLATION INSTRUCTIONS

Ubiflex is suitable for all flashing applications where lead is normally used.

Chimneys - Abutments - Rooflights – Pitched Valley Liners - Dormers

Steps & Saddles - Solar Panels - Cavity Trays.

Ubiflex is worked in the same way as lead, but without the need for

protective measures. It can be cut with a sharp knife or snips.

As Ubiflex is not susceptible to thermal movement, long runs can

be completed without the need for expansion joints.

- Lay Ubiflex granular side up.

Overlap joints (150mm), must be sealed with a continuous bead of  Ubiflex High-Tack sealant close to the edge and pressed down to ensure a good seal.

For flashing, fit Ubiflex into the pointing gap by at least 30mm and

hold in place with Ubiflex clips. Fold down and dress Ubiflex to

the roof covering, apply spots or a continuous bead of High-Tack

Sealant and press down to seal. Apply Ubiflex Gap-Seal into the pointing gap to ensure a waterproof joint and smooth as necessary. For non-brickwork areas, Ubiflex can be mechanically fixed back to the structure with non-ferrous fixings, if protected by an overhang. If unprotected, Ubiflex must be sealed to the structure with either High-Tack to prevent water ingress between Ubiflex and the structure.

If Ubiflex is likely to come into contact with aggressive chemicals during maintenance or other work, first test on Ubiflex before proceeding.

A minimum temperature of -10ºC is required for cutting and folding

and +5ºC for working with a lead dresser.

- Avoid foot traffic when installing valley lining or use a protection board.

- If further information is required please contact Ubbink.

Ubbink UK Ltd, Unit 33 Liliput Road, Brackmills Ind Est, Northampton, NN4 7DT

Tel: 01604 433000, email: info@ubbink.co.uk web: www.ubbink.co.uk